ECE3073 Project Test — Study Guide

What the test covers, how it is marked, and what to revise from each practical

# 1. The Test Format — Know This Cold

This is the single most important section. The mock test tells you exactly what to expect, so there are no surprises to manage — only preparation.

| Item | Detail |
| --- | --- |
| When | Week 12, in an allocated session (check your email for the time) |
| Duration | 1 hour 10 minutes total |
| Conditions | Closed-book. No notes, no internet, no generative AI. |
| Allowed materials | Only the provided zip: Intel NIOS II Embedded Peripherals Guide, ECE3073 Help Manual, µC/OS-II Manual, the Nios control-register macro .h file, the Prac 3 exception handler .c skeleton, the int-to-7-seg .c code, an archived Quartus project (.qar), and the RNG module Verilog file. |
| Two parts | (1) Practical build task — 9% of unit grade. (2) Git-related quiz — 3% of unit grade. |
| After the test | Demonstrate your working solution to a TA before leaving. Delete all test material. |

Key implication: the practical task is worth three times the Git quiz, so most revision time goes to the build task — but do not skip Git revision entirely, since 3% is easy marks if you know a handful of commands.

# 2. The Marking Breakdown — Where the 9 Marks Are

The practical task is marked as 9 separate point-scoring items. Each line below is one mark (or two). Treat this as a checklist: the goal is to make each line true. Study in order of how unsure you are about each — weakest first.

| Marking item | Marks | Comes mainly from |
| --- | --- | --- |
| Platform Designer system contains all required modules | 1 | Pracs 1, 2, 3, 5 |
| Platform Designer instantiated correctly + Verilog compiles | 1 | Pracs 1, 2 |
| System prints the required message to the Nios II console | 2 | Prac 5 (uC/OS-II), Prac 4 |
| System runs at least 2 meaningful uC/OS-II tasks | 1 | Prac 5 |
| System calculates and displays CPU usage every 5 seconds | 1 | Prac 5 |
| System reads switch values using interrupts correctly | 1 | Prac 3 |
| System compares values and reboots when needed | 1 | Prac 5, general logic |
| Semaphores used to remove computation from the ISR | 1 | Prac 5 |

Read this honestly: the heaviest cluster of marks (tasks, CPU usage, semaphores, the print message — 6 of 9) comes from Prac 5, the RTOS prac. Prac 5 is your highest-value revision target. Interrupts (Prac 3) and Platform Designer setup (Pracs 1–2) are the rest.

Important gap to fix: the Practical 5 PDF you have is mostly blank — only the intro pages rendered. Download the complete Practical 5 from Moodle before revising, because it is the prac the test leans on most.

# 3. The Build Task, In Plain Terms

Stripped of detail, the system you must build does this:

- On boot, print your name to the Nios II console.

- Read a random 8-bit number from the provided RNG module.

- Display that number on the LEDs.

- Wait for the user to enter the same number on the switches — switch input MUST be read using interrupts.

- When the switches match the random number, fetch a new random number and repeat.

- Throughout, run uC/OS-II with at least two meaningful tasks, print CPU usage every 5 seconds, and use a signalling semaphore to keep heavy work out of the interrupt service routine.

The shape of the solution: a Platform Designer system (Nios II/e, RAM, JTAG UART, PIOs for LEDs and switches, the RNG module, an interval timer for the RTOS) + a uC/OS-II C program with multiple tasks, an ISR for the switches, and a semaphore linking the ISR to a task.

# 4. What Each Practical Taught — and Why It Is on the Test

For each practical: the core skills, and how they feed the test. Rate yourself on each (see Section 6) and revise weakest-first.

## Practical 1 — Eclipse Project Setup & Disassembly

- Setting up an Eclipse Nios II project from a template; pointing it at the correct .sopcinfo file.

- Building, running, and downloading a program to the DE-board.

- Using the debugger: breakpoints, single-stepping, the Disassembly window, examining and editing registers and memory.

- Pseudo-instructions: source instructions that disassemble into different actual machine instructions.

- Feeds the test: the Eclipse project workflow, running on hardware, and debugging — you will set up and run a project under time pressure, so this must be automatic.

## Practical 2 — Nios Assembly, Qsys & Instruction Timing

- Building a Nios system from scratch in Quartus + Platform Designer (Qsys): adding the processor, on-chip memory, PIOs, making connections, assigning base addresses, setting reset/exception vectors.

- Writing the top-level Verilog module that instantiates the Qsys system and wires it to board pins (clock, KEY, LEDR, SW); importing pin assignments; adding the .qip file; compiling.

- Instruction timing: R-type, I-type, J-type and branch instructions, and clock cycles per instruction.

- Feeds the test: this is the core Platform Designer + Verilog instantiation skill — two marks depend directly on the system being built and compiling. The RNG module is wired in much like the PIOs here.

## Practical 3 — Event Handling: Polling vs Interrupts

- Polling: an infinite loop that repeatedly checks an input; simple but latency depends on loop length.

- Interrupts: configuring a PIO to generate an IRQ (Synchronously Capture, Generate IRQ, Edge type), connecting the IRQ line in Platform Designer.

- The four conditions for an interrupt to fire: three from the Nios II control registers (ienable, status/PIE, and the ipending bit), and one from the PIO's own interrupt mask.

- The exception handler and the interrupt_handler function; using the control-register macro header.

- Feeds the test: one mark is directly ‘reads switches using interrupts correctly’. The test explicitly requires interrupt-based switch reading, so interrupt setup is essential, not optional.

## Practical 4 — Multiprocessor Systems

- Multiple Nios processors on one board; unique CPU IDs; shared vs private peripherals.

- Hardware mutexes for mutual exclusion: open, lock, unlock; protecting shared resources.

- Inter-processor communication via shared memory and PIO-driven interrupts.

- Feeds the test: the test is single-processor, so multiprocessor specifics are unlikely to be built — but the idea of structuring code into separate concurrent units, and protecting shared data, carries directly into the uC/OS-II tasks-and-semaphores work. Lower priority than Pracs 3 and 5.

## Practical 5 — Concurrent Tasks & Inter-Process Communication (uC/OS-II)

- The uC/OS-II real-time kernel: creating and running concurrent tasks.

- Timer interrupt for the RTOS tick; measuring its overhead; the interval timer module with its own IRQ.

- Displaying processor utilisation (CPU usage %) and current time.

- Inter-process communication: signalling semaphores to pass work from an ISR to a task.

- Mutual exclusion with semaphores to protect shared data structures.

- Feeds the test: this is THE prac for the test — 6 of the 9 marks trace back here (running tasks, CPU usage, semaphores, the boot print). The PDF you uploaded is incomplete; get the full version from Moodle and treat it as your primary revision.

# 5. Lecture Theory & Tools — What Actually Helps This Test

## Theory weeks: only three matter for this test

The project test is a hands-on build, marked entirely on the build. Most lecture-theory weeks are FINAL-EXAM content, not project-test content, and should be left until after this test. Only three weeks give light conceptual support for the build:

- Week 4 — Devices (I/O), Interrupts, Busses. Most relevant. The test requires reading switches via interrupts; this week is the ‘why’ behind that mark. Review lightly if interrupts feel shaky.

- Week 8 — Operating Systems (Processes). Conceptual backing for the uC/OS-II task structure. Light skim.

- Week 9 — Concurrency: Coordination & Sharing Resources. Conceptual backing for semaphores and keeping work out of the ISR. Light skim.

- Everything else (Weeks 1–3, 5–7, 10–11 — pipelining, caches, scheduling, parallel processing) is final-exam material. Do NOT spend project-test time on it. Pivot to it after the test.

## Tools: practise on the real toolchain

- Hardware side — use Quartus and Platform Designer directly. There is no faster alternative, and the test runs on exactly these. Speed here comes only from repeated practice, not better tools.

- C code side — optionally write in VS Code, build in Eclipse. Nios II Eclipse is a slow editor; writing the uC/OS-II C in VS Code and pasting it into the Eclipse project to build is a legitimate speed-up. The toolchain stays the same.

- But verify closed-book. On test day there is no autocomplete or AI. Keep checking that I can write a task, an ISR, and a semaphore post/pend from memory — if a tool becomes a crutch, it has hurt me.

# 6. Pre-Test Readiness Checklist

Tick each box only when you can do it from memory, closed-book, in the way the test demands. Anything unticked is a revision target.

## Platform Designer / Hardware

- Build a Nios II/e system: processor, RAM, JTAG UART, timer, PIOs — from scratch, no notes.

- Assign base addresses and set reset + exception vectors correctly.

- Add a PIO and configure it to generate an Edge interrupt; connect the IRQ line.

- Integrate a provided Verilog module (the RNG) into the top-level design and wire it up.

- Write the top-level Verilog, import pin assignments, add the .qip, and compile with no errors.

## Interrupts (Prac 3)

- State the four conditions required for an interrupt to fire.

- Set up the control registers using the macro header to enable an interrupt.

- Write an interrupt_handler that services a PIO interrupt and clears the interrupt flag.

## uC/OS-II / RTOS (Prac 5)

- Create and run at least two uC/OS-II tasks with stacks and priorities.

- Print a message (your name) to the Nios II console on boot.

- Calculate and print CPU usage every 5 seconds.

- Create a signalling semaphore; post it from an ISR; pend on it in a task.

- Explain why heavy work belongs in a task, not the ISR.

## Git quiz (3%)

- Comfortable with core Git commands and concepts (revise from unit Git material on Moodle).

# 7. How to Revise This — Weakest-First

Apply the method from your study profile: do not let interest pick the order. Rate each practical 1–5 for how ready you feel, then revise from the lowest score upward.

| Practical | My readiness (1–5) | Revise? |
| --- | --- | --- |
| Prac 1 — Eclipse & debugging |  |  |
| Prac 2 — Qsys & Verilog instantiation |  |  |
| Prac 3 — Interrupts |  |  |
| Prac 4 — Multiprocessor (lower priority) |  |  |
| Prac 5 — uC/OS-II (highest priority) |  |  |
| Git commands |  |  |

- Practise the build, do not just read it. The test is a hands-on build. Closed-book, time yourself doing a full Platform Designer + Eclipse setup at least once before the test — setup speed is free marks and reading about it is not the same as doing it.

- Rehearse the timing. 70 minutes is tight: roughly, hardware build and compile first, then the uC/OS-II code, then the interrupt path, leaving a few minutes for the Git quiz. Know your order before you sit down.

- Explain it out loud. You demo to a TA and answer questions without AI. For each marking item, practise saying in one or two sentences what it does and why — this is both revision and demo rehearsal.

- Get the missing Prac 5. Your uploaded Prac 5 is incomplete. Download the full document from Moodle first — it is the most-tested prac and you cannot revise it from what you currently have.

Built from the 5 practical documents and the mock project test. Verify all details against Moodle and your unit guide.